Chapter V, Labour Code

## Health and Hygeine

51. Cleanliness. -

(1) Every establishment shall be kept clean and free from effluvia arising from any drain privy or other nuisance, and in particular, -

(a) accumulation of dirt and refuge shall be moved daily  by sweeping or by any other effective method from the  floors and benches of workrooms and from staircases and  passages and disposed of in a suitable manner;

(b) the floor of every workroom shall be cleaned at least  once in every week by washing, using disinfectant where  necessary or by some other effective method;

(c) where the floor is liable to become wet in the course  of any manufacturing process to such extent as is capable  of being drained, effective means of drainage shall be  provided and maintained;

(d) all inside walls and partitions, all ceilings or tops  of rooms and walls, sides and tops of passages and  staircases shall-

(i) 	where they are painted or varnished, be  repainted or revarnished at least once in every  five years;

(ii) 	where they are painted or varnished and have  smooth impervious surfaces, be cleaned at least  once in every fourteenth month, by such methods  as may be prescribed;

(iii) in any other case, be kept white- washed or  colour-washed and the white-washing or colour-washing  shall be carried out at least once in every  fourteen months; and

(e) the dates on which the processes required by clause (d)  are carried out shall be entered in the prescribed  register.

52. Ventilation and temperature. -

(1) 	Effective measures shall be made in every establishment for securing and maintaining in every work-room adequate ventilation by the circulation of fresh air;  and

(2) 	Temperature in every room shall be kept in such conditions as will secure to workers therein  reasonable conditions of comfort and prevent  injury to health	◦

(3) 	For the purpose of sub-section (2), the walls and roofs of rooms shall be of such material  and so designed that such temperature shall not  be exceeded but be kept as low as practicable;

(4) 	where the nature of the work carried on in  the establishment involves, or is likely to involve,  the production of excessively high temperature,  such adequate measures as are practicable, shall  be taken to protect the workers therefrom by  separating the process which produces such temperature from the work-room by insulating the  hot parts or by other effective means.

(5) 	If it appears to the Government that in any establishment or class or description of factories excessively high temperature can be reduced by such methods as white-washing, spraying or insulating and screening outside walls or roofs or windows or by raising the level of the roof, or by insulating the roof either by an air space and double roof or by the use of insulating roofing materials, or by other methods, it may prescribe such of those or other methods to be adopted in the establishment

53. Dust and fume. -

(1) 	In every establishment in which, by reason of the manufacturing process carried on, there is given off any dust or fumes or other impurity of such a nature and to such an extent as is likely to be injurious or offensive to the workers employed therein, effective measures shall be taken to prevent its accumulation in any work- room and its inhalation by workers, and if any exhaust appliance is necessary for this purpose, it shall be applied as near as possible to the point of origin of the dust, fumes or other impurity, and such point shall be enclosed so far as is possible.

(2) 	In any establishment no stationary internal combustion engine shall be operated unless the exhaust is conducted into open air, and no internal combustion engine shall be operated in any room unless effective measures have been taken to prevent such accummulation of fumes therefrom as are likely to be injurious to the workers employed in the work-room.

54. Disposal of wastes -

(1) Effective arrangements shall be made in every establishment for the disposal of wastes and effluents due to the manufacturing process carried on therein.

55. Artificial humidification. -

(1) 	In any establishment in which the humidity of the air is artificially increased, the water used for the purpose shall be taken from a public supply, or other source of drinking water, or shall be effectively purified before it is so used.

(2) 	If it appears to an Inspector that the water used in an establishment for increasing humidity which is required to be effectively purified under sub-section (1) is not effectively purified, he may serve on the Manager of the establishment an order in writing, specifying the measures which in his opinion should be adopted, and requiring them to be carried out before a specified date.

56. Overcrowding

(1) 	No work-room in any establishment shall be overcrowded to the extent that it is injurious to the health of the workers employed therein.

(2) Without prejudice to the generality of the foregoing provisions, there shall be provided at least 9.5 cubic metre of space  for every worker employed in a workroom

Explanation - For the purpose of this sub-section  no account shall be taken of a space which is more than  fourteen feet above the level of the floor of the room.

(3) 	If the Chief Inspector by order in writing so requires, there shall be posted in each work-room of a establishment an establishment specifying the maximum number of workers who may, in compliance with the provisions of this section, be employed in the room.

(4) 	The Chief Inspector may, by order in writing exempt, subject to the conditions as he may think fit to impose, any work- room from the provisions of this section if he is satisfied that compliance therewith in respect of such room is not necessary for the purpose of health of the workers employed therein.

57.  Lighting. -

(1) 	In every part of an establishment where workers are working or passing, there shall be provided and maintained sufficient and suitable lighting, natural or artificial, or both.

(2) 	In every establishment all glazed windows and skylights used for the lighting of the work-room shall be kept clean on both the outer and inner surfaces and free from obstruction as far as possible under the rules framed under sub-section (3) of section 14.

(3) 	In every establishment effective provision shall so far as is practicable, be made for the prevention of -

(a)  glare either directly from any source of light or by  reflection from a smooth or polished surface, and

(b)  the formation of shadows to such an extent as to  cause eye strain or risk of accident to any worker.

58. Drinking water. -

(1) 	In every establishment effective arrangements shall be made to provide and maintain at a suitable point conveniently situated for all workers employed therein, a sufficient supply of wholesome drinking water.

(2) 	All such points shall be legibly marked "Drinking Water" in Bengali language

(3) 	In every establishment wherein more than two hundred and fifty workers are ordinarily employed, provision shall be made for cooling the drinking water during the hot weather by effective means and for distribution thereof.

(4) 	Effective Arrangement shall be made for oral dehydration therapy for those worker who may lose water from their bodies for working near or around machineries producing excessive heat.

59. Latrines and urinals. -

(1) In every establishment -

(a) sufficient latrines and urinals of prescribed types  shall be provided conveniently situated and accessible to  workers at all times while they are in the establishment;

(b) enclosed latrines and urinals shall be provided  separately for male and female workers;

(c) such latrines and urinals shall be adequately lighted  and ventilated and no latrine or urinal shall, unless  specifically exempted in writing by the Chief Inspector,  communicate with any work-room except through an  intervening open space or ventilated passage;

(d) all such latrines and urinals shall be maintained in  a clean and sanitary condition at all times with suitable  detergents or disinfectants or with both;

# 21. Spittoons. -

(1) 	In every establishment there shall be provided, at convenient places, a sufficient number of spittoons which shall be maintained in a clean and hygienic condition.

(2) 	No person shall spit within the premises of an establishment except in the spittoons provided for the purpose.

(3) 	A notice containing this provision and the penalty for its violation shall be prominently displayed at suitable places in the premises.